Raingarden construction checklist

### This list has been developed by Melbourne Water for use by Councils in assessing raingarden construction projects.

| Raingarden location |  |
| --- | --- |
| Inspected by |  |
| Asset ID |  |
| Date and time |  |

| Item Inspected | Satisfactory (Y/N/NA) | Comments |
| --- | --- | --- |
| Preliminary Works |  |  |
| 1. Erosion and sediment control plan adopted |  |  |
| 2. Traffic control measures |  |  |
| 3. Location including inlet same as plans |  |  |
| 4. Site protection from existing stormwater flows |  | Hold Point |
| Earthworks and structural works |  |  |
| 5. Bed of basin correct shape |  |  |
| 6. Batter slopes and edging as per plans |  |  |
| 7. Dimensions of filter and surface area as plans |  |  |
| 8. Confirm surrounding soil type with design |  |  |
| 9. Impermeable 1mm PVC liner, hot weld all joints |  |  |
| 10. Pipe joints and connections as designed |  |  |
| 11. Inlet, perforated pipe underdrain, flushing points and overflow pit installed as designed |  | Hold Point |
| 12. Drainage layer media as designed |  |  |
| 13. Transition layer media as designed |  |  |
| 14. Filter media specifications tested from stockpile delivered to site. Hydraulic Conductivity, Particle Size Distribution and Organic Content are within Specifications | Y/N |  |
| 15. Surface level of filter media is correct, leaving ponding depth as per drawings. Surface is entirely flat |  | Hold Point |
| 16. Surface mulch is washed stone aggregate, distributed with space around plant stems. |  |  |
| 17. Planting as designed (species and densities) |  | Hold Point |