Swale design assessment checklist

This list has been developed by Melbourne Water for use by Councils in assessing capital works and developer constructed swales.

| Location |  |
| --- | --- |
| Catchment area (ha) |  |
| Swale area (m2) |  |
| Minor flood (m3/s) |  |
| Major flood (m3/s) |  |

| Design stage | Treatment | Y/N | Where to look |
| --- | --- | --- | --- |
| Concept | Treatment performance verified by MUSIC? |  | Design report |
|  | Hydraulics |  |  |
| Concept | Station selected for IFD appropriate for location? |  | Design report |
| Functional | Longitundinal slope of swale invert between 1% to 4%? |  | Drawings |
| Functional | Manning's 'n' selected appropriate for proposed vegetation type? |  | Calculations |
| Functional | Overall flow conveyance system sufficient for design flood event? |  | Calculations |
| Detailed | Overflow pits provided where flow capacity exceeded? |  | Drawings |
| Functional | Velocities within swale cells will not cause scour? |  | Calculations |
| Detailed | Inlet to swale set down of at least 50 mm to prevent blocking with sediment? |  | Drawings |
| Functional | Maximum ponding depth and velocity will not impact on public safety (v x d <0.4)? |  | Calculations |
| Detailed | Maintenance access provided to invert of swale? |  | Drawings |
| Design Stage | Plants | Y/N | Where to look |
| Detailed | Plant species selected can tolerate periodic inundation and design velocities? |  | Detailed |
| Detailed | Plant species selected integrate with surrounding landscape design? |  | Detailed |